31 Mar 2006

To:	WaveTrain users and developers

From:	Boris Venet, MZA Associates (venet@mza.com,  505-245-9970)

Re:	Validation of PartiallyCoherentReflector model in WaveTrain

Abstract

Speckle statistics for polychromatic light scattered from an optically rough reflector differ from the familiar exponential irradiance statistics found in monochromatic speckle.  We derive an analytic result for the polychromatic speckle irradiance contrast for a simple reflector geometry, as a function of the geometry parameters and the coherence or spectral density parameters of the incident light.  We then use this result to set up a quantitative test of WaveTrain’s “PartiallyCoherentReflector” module, whose purpose is to numerically simulate the behavior of polychromatic (narrow-band) light scattered from an optically rough reflector.

Validation of “PartiallyCoherentReflector” model in WaveTrain

Boris Venet,  MZA Associates
31 Mar 2006

# Introduction

The purpose of WaveTrain’s “PartiallyCoherentReflector” (PCR) module is to numerically simulate the reflection of light from an optically rough surface, when the incident light has a finite longitudinal (temporal) coherence, or equivalently put, is polychromatic to a certain extent.  It is desired with the PCR numerical model to accurately simulate the propagation and speckle behavior when the longitudinal coherence varies over the domain from a few object macroscopic depths to a relatively small fraction of the macro-depth.  The most obvious qualitative effect of the finite coherence is to reduce the contrast of the rough-surface speckle observed in a receiver plane.  The reduction occurs because reflector points that are separated in macro-depth by more than the longitudinal coherence length will no longer produce stable interference:  rather, the irradiances due to those reflector points will simply add.

Qualitatively, previous exercise of the PCR model showed that it certainly decreased speckle contrast, but the results had never been checked in any quantitative sense, specifically by comparison with analytically-derived results for a tractable geometry.  There are several questions that need exploration as far as the numerical simulation model is concerned.  Two key questions are:  (a) Does the PCR simulation model actually give quantitatively correct speckle behavior for different combinations of target geometry and coherence?   (b) How many “speckle realizations” are necessary to make the simulation model approach the correct statistics for given geometry/coherence parameters?

The present paper has the following purposes:
(a)  We derive an analytic result for the speckle irradiance contrast for a simple reflector geometry, as a function of the geometry parameters and the coherence or spectral density parameters of the incident light.
(b)  We review the concept used by WaveTrain's PCR numerical model.
(c)  Using the results from (a), we set up a test case for checking the operation of the PCR wave-optics simulation model.

# Theoretical speckle contrast for a test case

## Geometry and monochromatic irradiance

Consider the geometry shown in Figure 1.  Let the z axis be the nominal direction of the incident illumination, and let ρ=(x,y) be the transverse coordinate.  The incident beam is scattered by an optically rough surface, which has an arbitrary orientation with respect to the axes.  Let the plane z = Z be a reference plane from which the surface depth function d(ρ) is measured.  This depth function refers only to the macroscopic surface shape, while the extra perturbation h(ρ), a random process, designates the surface micro-roughness.  We will begin with a spatially discrete model, using a single subscript to designate the two-dimensional grid point location.  We will use the equivalent notations d(ρm)=dm, h(ρm)=hm, and so forth as is convenient.  Let the plane z = 0 be the observation plane in which we want to compute speckle irradiance statistics.  For single-point statistics it is sufficient to consider one observation point, P, which we place at the origin.

Figure 1:  Scatter from an optically rough surface with depth profile

Our interest is in formulating the problem for polychromatic light, but we can build up that result by beginning with the more familiar monochromatic case.  The key starting point is a formula for the net complex field at the observation point P.  Using the discrete spot model of the surface, the net field at P can be expressed as the superposition of phasors that represent the phase lags from the reference plane to each discrete scatter point and then back to P.  Let the incident complex field at z = Z  (monochromatic, with wave number k=(2π)/λ ) be given by

Vm0(k)= Am0k e iθm0(k) e−i (ck) t=Um0(k) e−i (ck) t

The optical angular frequency factor written with ω=ck means that we assume dispersion is insignificant for our problem.  (In purely monochromatic analysis, we typically drop the exp(−iω t) factor completely.)  Now let Rm be the intensity reflectance, let ℓmbe the physical path length from the reference plane to the scatter point and then to P, and let g be a geometry factor that represents the spread of the scattered light due to the back-propagation.  With these definitions, the net complex field at P is

V(k)=m=1M g Rm ei k ℓm Um0k e−i c k t=U(k) e−i c k t

Equation  is a discrete version of the Fresnel-Kirchoff diffraction integral.  We will also need an explicit decomposition for ℓm in terms of its several elements, namely

ℓm≈dm+hm+Z+dm+hm+ρm22ZZ+2dm+2hm+ρm22Z

The quadratic approximation of Equation  is consistent with the usual Fresnel approximation.

Optical sensors are square-law devices that output a signal proportional to the time-average of the square of the electric field, where the averaging time is the response or integration time of the sensor.  We will use the symbol I(k) to represent this irradiance.  As shown in any general optics textbook, this time average for the monochromatic case turns out to be equivalent to the simple expression

I(k)=U(k) U(k)

The only requirement for Equation  to hold is that

(integration time)>>1/ (optical frequency)	.

## Transition to the polychromatic case (finite bandwidth or finite longitudinal coherence)

The polychromatic field can be constructed from a Fourier superposition of monochromatic field terms defined in Equation :

V = dk V(k) = dk m=1M g Rm ei k ℓm Um0(k) ei c k t

Next, we need an expression for the irradiance of the polychromatic field V.  Computing this requires careful specification of the nature of the k dependence in Um0(k).  Unless one has a special known source model, one considers the polychromatic field amplitude to be a random process in the k space.  To begin with, when we only deal with longitudinal coherence, it is useful to factor Um0(k) as

Um0(k)=um0⋅u0(k)

where the  u0(k) factor is a random process and the um0 is a deterministic term that carries the space dependence.  We assume that the deterministic factor um0 is constant over the spectral bandwidth.  The fact that the same u0(k) applies to all the space points is the meaning of finite longitudinal coherence.  The factorization  yields

V=dk u0(k) ei c k t m=1M g Rm um0 ei k ℓm =dk u0(k) ei c k t  U(k)

where the second line defines the new symbol U(k).

We now skip a few derivation details and assert that the irradiance of the V field, under a certain condition on the optical bandwidth, is given by

I=dk S(k) U(k) U(k) where   S(k)=u0(k) u0(k)

The function S is called the spectral density, and the time-average overbar can alternately be viewed as an ensemble expectation value with respect to the ensemble of possible realizations of the spectral random process.  The key condition that leads to Equation  is that

(integration time)>>1/ (frequency bandwidth of S)

The above mathematical model for polychromatic speckle follows the basic formulation given by Parry ().

To reduce the future formula burden somewhat, we note that there is no loss of generality if we assume that S is normalized so that

dk S(k)=1

In the cases of interest to us, S(k) is a relatively narrow function concentrated around some kavg.

## Spatial moments of the irradiance

It is important to note that Equation  for the polychromatic irradiance I has taken care of one random process present in the V field, namely the random fluctuations in magnitude and/or phase of the source field.  We are assuming, because of condition , that I will not experience any noticeable temporal fluctuations due to the source random process (on the measurement time scale).  There remains, however, another random process in I, namely the hm contained in the ℓm terms.  A given realization of  the hm process produces a given, temporally fixed, spatial distribution of I in the observation plane:  this is one realization of the polychromatic speckle.  Different realizations of hm would produce different distributions of  I across the observation plane.  We wish to compute a 1-point statistic, namely the spatial variance of the irradiance.  We will compute this from the ensemble moments

Ih   and   I2h

where the subscript emphasizes that this expectation value is with respect to the surface height random process only.  The final quantity of interest is actually the normalized irradiance variance (NIV), that is, [variance / mean2 ].  Alternatively, the “contrast” is usually defined as [standard deviation / mean] = NIV.  Once we have Ih and I2h, we can easily combine these to form the NIV or contrast.

We said above that I has no measurable temporal fluctuations due to the source phase random process.  However, the width of the spectral density will still affect the calculation of the second spatial moment of I.  Qualitatively this comes about because Equation , for any given height realization, adds speckle irradiances corresponding to different wave numbers and these patterns are slightly displaced from one another and therefore smear the monochromatic contrast.

## Evaluation of Ih

Combining Equations ,  and , we have

Ih=dk S(k) U(k) U(k)hdk S(k) g2 m, n Rm Rn um0 un0 ei 2k dm−dn+ρm2−ρn24Z ei 2k hm−hn

We can evaluate the expectation value factor containing the surface heights, if we assume that the heights are Gaussian, independent, identically distributed random variables.  This is the typical assumption made in the scatter spot model for speckle.  In that case, the expectation value becomes

ei 2k hm−hn=ei 2k hm−hn e−12⋅4k2σ2(hm−hn)=e−4k2σh2 , m≠n

where σh2 is the surface height variance.  (The basic result for <exp(i*Gaussian)> is given, for example, in Goodman ()).

The factor exp−4k2σh2 can be rewritten as exp−16π2⋅(σh/λ)2.  So, if the surface height standard deviation is of order 1 wavelength or more, the exp(…) factor is of order e−160≈3⋅10−70 or less.  Substituting Equation  into Equation  yields a result comprising two classes of terms, corresponding to the two sums in the following equation:

Ih=g2dk S(k)m Rm  um02 +e−4k2σh2m≠n Rm Rn um0 un0 ei 2k dm−dn+ρm2−ρn24Z

Using the fact that x1+…+xM≤x1+⋯+xM and that exp−4k2σh2 is miniscule as discussed above, we can neglect the second term in Equation  compared to the first (even though the second term contains M(M−1) summands, compared to the first term’s M summands).  Thus, Ih reduces to

Ih=g2dk S(k) m Rm  um02g2 m Rm  um02

To obtain the second line of Equation , we used two facts:  um02 changes negligibly over the spectral bandwidth, and S is normalized according to Equation . Contemplating the result , it may now seem obvious from the standpoint of energy conservation.  We went through the trouble of showing the intermediate steps because manipulations of the same general type, but with more algebraic complications, will be used to obtain the second moment I2h.

## Evaluation of I2h

Again using Equations ,  and , we have

I2h=dk1 S(k1) U(k1) U(k1) ⋅ dk2 S(k2) U(k2) U(k2)dk1 dk2 S(k1) S(k2)g4m,n,p,qRm Rn Rp Rq um0 un0up0 uq0 ei 2k1 dm−dn+ρm2−ρn24Z+k2 dp−dq+ρp2−ρq24Z ei 2k1 hm−hn+k2 hp−hq

Using the same concepts employed earlier, we can evaluate the exponential expectation value (i.e., the last factor) in Equation .  Defining φ=k1 (hm−hn)+k2 (hp−hq), we have

ei 2 φ=ei 2 φ e−12⋅4 σφ2

The heights are assumed zero-mean and identically distributed.  Therefore the first factor in Equation  is simply 1, while in the second factor the variance of φ reduces to

σφ2=k12 hm2+hn2−2hmhn+k22 hp2+hq2−2hphq+2 k1k2 hmhp−hnhp−hmhq+hnhq

Writing

hj2=σh2  for all j,   andhmhn=δmn σh2

where δmn is Kronecker’s delta,  Equation  can be conveniently rewritten as

σφ2=2 σh2⋅ k12 1−δm n+k22 1−δp q+k1 k2 δmp−δnp−δmq+δnq

Consider now various possible cases of equality and inequality among {m, n, p, q}.  The various Kronecker deltas always take on the values 0 or 1.  The following list shows three particular combinations:

m=n  and p=q: σφ2=0  ,   and  e−2σφ2=1

m=q  and  n=p,  but  m≠n: σφ2=2 σh2 k1−k22  ,   and  e−2σφ2=e−4 σh2 k1−k22

m≠n  and  p=q: σφ2=2 σh2 k12  ,   and  e−2σφ2=e−4 σh2 k12

The exponential factor at the far right of the preceding equations is what appears in Equation  and hence in the terms of the master Equation .  We have met the exponential factor of case  before, when we evaluated Ihand we noted its miniscule size (recall Equation  and the subsequent discussion).  Careful examination of all other subscript equality-inequality cases shows that all the cases not shown in the above list have the general character of case .  Therefore, the only two terms that may substantially contribute to Equation  are cases  and .

Using the results of the previous paragraph, Equation  reduces to

I2h=dk1dk2 S(k1) S(k2) g4m,p Rm Rp  um02  up02                                            +g4m,n Rm Rn  um02  un02 ei 2 k1−k2 dm−dn+ρm2−ρn24Z e−4 σh2 k1−k22T1+T2

Inspection of Equation  shows that the first term of Equation , T1, is simply

T1=Ih2

To evaluate T2, we carry out the wave number integrations first.  Since the integrand depends only on (k1−k2), we change integration variables from (k1,k2) to sum and difference coordinates (K=k1+k2/2, κ=k1−k2).  Making the change of variables and then doing the K integral first yields

T2k≡dk1dk2 S(k1) S(k2) ei 2 k1−k2 dm−dn+ρm2−ρn24Z e−4 σh2 k1−k22=dκ dK S(K+12κ) S(K−12κ) ei 2 κ dm−dn+ρm2−ρn24Z e−4 σh2 κ2=dκ BS(κ) e−4 σh2 κ2 ei 2 κ dm−dn+ρm2−ρn24Z

The new symbol BS(κ) in the last line of Equation  is the deterministic autocorrelation of S(k), which appeared in the square brackets of the preceding line.  Note also that the last line of Equation  can be interpreted as an inverse Fourier transform, evaluated at the space variable ξm,n, where ξm,n=2 dm−dn+(ρm2−ρn2)/4Z.  That is, it may be helpful to write

T2k=Fκ−1  BS(κ) e−4 σh2 κ2 | ξ m,n

The full T2 term of Equation  is therefore

T2=g4m,n Rm Rn  um02  un02 Fκ−1  BS(κ) e−4 σh2 κ2 | ξm,n

It is important to realize that the exponential factor exp−4κ2σh2 in Equation  is a very different animal than the previously encountered factor exp−4k2σh2, which we argued was negligible.  The reason is that κ is a difference wavenumber, which ranges over only a small increment around 0, whereas k was a wave number ranging over an increment around the average optical wave number.

## NIV

Finally we can obtain our end goal, the speckle NIV, by combining the Equations  and  for T1 and T2:

NIV = I2h−Ih2Ih2 = (T1+T2)−T1T1 m,n Rm Rn  um02  un02 Fκ−1  BS(κ) e−4 σh2 κ2 | ξm,nm,p Rm Rp  um02  up02

Formula  is almost as far as we can reduce the answer in general.  To proceed much further and obtain numerical answers, we must specify explicit functional forms for the spectral density function, the incident spatial field dependence, and the reflectance spatial dependence.  In the next section, we work out a numerical example that can be used for detailed testing of the WaveTrain PartiallyCoherentReflector module.

Before proceeding to a complete numerical example, there is actually one more approximation we can make that will frequently be very good, at least when the illuminator is a laser.  The factor exp−4κ2σh2 in Equation  will often be very nearly equal to 1, over the entire κ domain for which the spectral density (and hence the autocorrelation BS(κ)) is non-negligible.  To see this, we write

4 κmax σh2=4 (Δk)max⋅ σh2≈4 2π(Δf)maxfavg σhλavg2= 4π(Δf)maxfavg σhλavg2

Evidently, if

(Δf)maxfavg << 1 /4πσhλavg

then exp−4κ2σh2≈1.  Table 1 shows some examples that indicate this relation will frequently be satisfied.  The table assumes λavg=1μmand correspondingly favg=3⋅1014Hz:

| (Δf)maxfavg | 1 /4πσhλavg |
| --- | --- |
| 1GHz / 3E14Hz = 3E-6 | 1/(4 1) = 8E-2 |
|  | 1/(4 100) = 8E-4 |
| 1MHz / 3E14Hz = 3E-9 | 1/(4 1) = 8E-2 |
|  | 1/(4 100) = 8E-4 |

Table 1:  Numerical examples related to Equation

From the tabulated examples, it seems that inequality  will typically be satisfied for laser illumination.  When the inequality is satisfied, the NIV result  simplifies further to

NIV = m,n Rm Rn  um02  un02 Fκ−1  BS(κ)  | ξm,nm,p Rm Rp  um02  up02

## NIV evaluated numerically for specific functional forms

Consider the following example:

- The reflector is a rectangular plate with dimensions L1×L2.  The L1 edges are parallel to the x axis, and the L2 edges are tilted at angle θ with respect to the y axis (see Figure 2).  Therefore, the mean depth function of the plate is dm=ymtanθ.  The total depth of the plate is Lz=L2sinθ, and the projected y-width is Ly=L2cosθ, as sketched below.

Figure 2:  Tilted rectangular plate geometry

- The reflectance is uniform.

- The spatial dependence of the incident field is a plane wave with wave vector along z.

- The spectral density function has a Gaussian shape, given by

S(κ)=2π Dκ2 e− 2 κDκ2

Note the bandwidth Dκ is defined as the 1/e2 half-width, and S(κ) is area-normalized to 1.

- The bandwidth Dκ satisfies Equation , so that NIV is given by Equation .

Evaluation of the spectral autocorrelation and Fourier transform required by the NIV formula yields

Fκ−1  BS(κ)  | ξm,n=e− Dκ dm−dn+ρm2−ρn24Z 2

Finally, we substitute this into Equation  and compute the remaining sums.  When we derived Equation  it was convenient to use a spatially discrete model for the random surface (Goodman’s “scatter-spot” model).  But, at the present stage it is more convenient to transition to a continuum model.  This means replacing each sum in Equation  by an area integral.  It is convenient to express the support domain of the reflectance factors by using the rectangle function rect(t)= 1  if t≤12, 0  otherwise.  The numerator of Equation  then becomes

num NIV=R2  u04 d2ρ rectxL1 rectyLy d2ρ' rectx'L1 recty'Ly e− Dκ y−y' tan θ + x2+y2−x−y4Z 2

Now we make a final approximation:  except for plate tilts θ≈0, the quadratic terms in the real exponential of the preceding integral are completely negligible compared to the linear terms.  In that case, the x integrals become trivial, and the y integrals can be simplified by changing to sum and difference coordinates.  The final result is the simple expression

NIV = num NIVden NIV = dt tri(t)⋅ e− Dk Lz t2

where the triangle function tri(t) is 1 at  t = 0 and drops linearly to 0 at t=±1.  This final integral must be evaluated numerically, and the result is shown as the red solid curve in Figure 3.

Figure 3:  Speckle NIV for the tilted rectangular plate, when illumination has finite bandwidth

The NIV depends only on the product parameter Dκ Lz, where Lz is simply the total depth of the plate (the actual length and tilt only enter as the combination Lz=L2sinθ).  Recall also that NIV is actually not exactly 1 when Lz= 0, since we have neglected the quadratic terms in Equation .  When Dκ Lz is large enough, the Gaussian factor in Equation  is very narrow compared to the triangle factor.  In that asymptotic case, the integral can be evaluated analytically, and we find that NIV≈π/(DkLz).  Smoothly merging the two asymptotes with an empirical joining function, we can obtain a simple semi-empirical formula that accurately represents the computed results over the whole domain (see the dots in  Figure 3):

NIV≈1+1πDκLz2 − 1/2

Notice that NIV decreases quite slowly, proportional to Dκ Lz−1.  The contrast decreases even more slowly, proportional to Dκ Lz−1/2.

To see what the NIV or contrast numbers are for a few laser bandwidths, consider sources with bandwidths of Df=10 MHzand 10 GHz, and let Lz=1 m.  Since k=2πf/c, the differential implies Dκ=2π/cDf, that is Dκ=0.21 rad/m and 210 rad/m for the two cases.  From Figure 3 or Equation , the NIV for the two cases is 0.993 and 0.0084, while the contrast is 0.996 and 0.092 .

### Coherence length

Since the wave number superposition was fundamental in the theoretical analysis, the spectral density width Dκ was a natural width measure to use in our development of the NIV result.  But there is another bandwidth measure that frequently arises in experimental practice, namely the “coherence length”, ℓcoh, of the illuminator.  Coherence length is typically measured via fringe visibility in a Michelson interferometer setup.  The coherence length is defined as the path-length difference that reduces fringe visibility from 1 to some specified level.  Since coherence length in this sense is frequently used or specified to characterize sources, it is important to relate this quantitatively to the Dκ width measure.  It can be shown that the fringe visibility as a function of path length difference is the Fourier transform of the spectral density function.  Therefore, for the Gaussian spectral density in our numerical example, i.e., Equation , the visibility function is also Gaussian, and the 1/e2 coherence length is related to Dκ (which was also a 1/e2 width) by the exact relation

ℓcoh=4Dκ

With this result for coherence length, we can rewrite Equation  as

NIV≈1+4πLzℓcoh2 − 1/2

When Lz/ℓcoh>>1, NIV→π/4⋅ℓcoh/Lz.  If we assume a qualitative model in which the scatter from successive "coherence depths" adds incoherently, then the proportionality NIV∝ℓcoh/Lz can be derived very simply from the addition of independent random variables.  This provides a check on the rather complicated derivation leading to Equations  and

From Equation , the two bandwidthsDκ=0.21 rad/m and 210 rad/m for which we computed NIV and contrast in the previous subsection correspond to ℓcoh≈19 m and 1.9 cm respectively.

### Cautionary comments

The precise numerical values of the breakpoints in the NIV Equations  and  depend on the assumption of Gaussian spectral density.  For a different spectral shape, say a Lorentzian, these breakpoints will be somewhat different, although the asymptotic slopes are most likely the same.  In practice, when a source is said to have a certain coherence length, it is often murky at best whether a 1/e2 or ½ or some other visibility level is meant.  With lasers, the situation is often also complicated by the fact that the shape of the visibility function is not an analytically simple form.  In practice then, we expect the proportionality NIV∝ℓcoh/Lz indicated by Equation , but the exact proportionality factor will be in doubt.

# Review of WaveTrain's “PartiallyCoherentReflector” numerical model

WaveTrain's PCR simulation model works directly with a space-time statistical representation of the partially coherent illumination.  The simulation procedure does not exactly parallel the spectral superposition approach that we used in Section 2 to work out a theoretical NIV.  However, the space-time approach should be equivalent:  if we use corresponding spectral density functions or temporal correlation functions, then the simulation calculation should yield results identical to what we computed theoretically in Section 2.

WaveTrain's PCR model works as follows.  Consider the sketch in Figure 4.  The curved line represents an optically rough reflecting surface, which is illuminated by a wave traveling paraxially in the z direction.  z0 is a reference plane from which the macroscopic depth map, d(x,y), of the target surface is specified.  The micro-roughness of the surface, in the optical sense, is a spatially random fine-scale perturbation on top of the macroscopic d map.

Figure 4:  Simulation of rough-surface reflection with partial longitudinal coherence

The PCR concept is an elaboration of the method used to treat the scattering of completely coherent illumination.  For completely coherent light (WaveTrain's "CoherentTarget" module), we represent the reflected complex field that exits the discrete mesh in the z0 reference plane as

Ui,jrfl=α Ri,j Ui,jinc eiδi,j

where Uinc(xi,yj) is the incident complex field, δ(xi,yj) is a spatially uncorrelated phase, R(xi,yj) is an intensity reflectance map, and  is a radiometric normalization factor required because of our discrete Fourier Transform methods (the  factor is discussed in other documentation, and is not specifically a PCR property).  The mod-2  values of are assumed for practical purposes to be uniformly distributed over (0, 2).  This model (, ) is sometimes called the scatter-spot model.  In this basic model, the deterministic depth map has no effect on the statistical properties of the scattered light recorded in the receiver plane, and is ignored in the formalism.  Now consider a beam with finite longitudinal coherence.  Most cases of interest are still "quasi-monochromatic", such that the slowly-varying envelope (SVE) representation is a useful mathematical device.  The incident field is described by

Ψ(ρ,z,t)=U(ρ,z,t) e ik0z − ω0t

where ρ=(x,y), k0 and ω0 refer to the nominal optical wavelength λ 0, and U  is the slowly-varying complex envelope factor.  In general, WaveTrain operates only on the U  factor, and does not keep track of the “carrier” phasor  e ik0z − ω0t.  In most WaveTrain operations, the t dependence of U arises only from turbulence and/or relative component motion.  However, in the PCR model, we explicitly construct a space-time representation of the SVE envelope:  this envelope is notionally pictured at one instant as the irregular wave in the Figure 4 sketch.  The first key operation of the PCR model is to construct one instantaneous realization of the partially-coherent SVE envelope factor AS(z)expiφS(z), assuming that we have a Gaussian random process with correlation length ℓcoh, which is a user input of the model.  The realization is constructed over a total length LS, on a mesh with spacing dzS. LS is also a user input of the model.  The next step is to construct one realization of the reflected complex field array, Urfl(xi,yj), exiting the discrete propagation mesh in the z0 reference plane.  By extension from Equation ,  we write

Ui,jrfl=α Ri,j Ui,jinc AS(z0+2di,j) eiφS(z0+2di,j) ei δi,j

The realization Ui,jrfl is then propagated away from the z0 plane towards the receiver using the same Fourier methods as for any completely coherent field.  One such realization would give the same statistical properties (fully-developed speckle) at the receiver as the basic coherent-illumination (CoherentTarget) model described by Equation .  To obtain the desired partially coherent result, we now repeat the propagation step with a sequence of Nw realizations of Ui,jrfl.  Each realization is obtained from Formula  by the replacement

AS(z0+2di,j) eiφS(z0+2di,j) → AS(z0+n⋅ΔzS+2di,j) eiφS(z0+n⋅ΔzS+2di,j)

where n=0,…,Nw−1, and ΔzS=LS/Nw.   {ℓcoh,LS, and Nw} are the three SVE-related user inputs of the PCR model.  When the Nw realizations are propagated to the receiver plane, this procedure represents an actual temporal sequence of complex fields that a receiver would experience due to the SVE illumination.  The temporal sequence of irradiance patterns at the receiver plane consists of speckle patterns that are somewhat shifted (and distorted) from one another.  Finally, the PCR model assumes that the response time of the sensor is slow enough so that only the average irradiance over the Nw realizations is reported by the sensor.  If Nw sufficiently samples all the correlation lengths contained within the target macro-depth L z, then this discretely-computed average should exhibit the speckle contrast reduction appropriate to the specified combination of coherence length and target macro-depth.

Figure 5 may help to clarify the required combination of parameters that provides sufficient sampling for validity of the PCR algorithm.  The key parameters in question are the SVE-related input parameters {ℓcoh,LS, and Nw}.  The curved line in Figure 5, with known total depth Lz, represents the rough reflecting surface.

Figure 5:  Choosing consistent parameters for the WaveTrain PCR model

The distance LS is the length (in meters) of the space-time realization of the SVE random process.  This realization is generated by an algorithm that produced a periodic realization, so we recommend setting LS = 2Lz.  If we sample the SVE envelope at least several times per correlation length, we should sufficiently resolve that variation to model any variations in the resulting speckle pattern.  We express this requirement as

LSNw<ℓcohseveral ⇒ Nw>(several)LSℓcoh

Next, consider the distance labeled dℓ in Figure 5.  This depth increment corresponds to the projection of the propagation mesh spacing dxyonto the reflecting surface, where θy is the local inclination of the surface with respect to the y-axis.  The sketch shows a y-z projection, but there is an analogous projection for x-z.  Again, in order for the numerical procedure to sample the SVE correlation length interacting with the surface, we should have dℓ less than some fraction of ℓcoh.  We express this requirement as

dℓ<ℓcohseveral ⇒ dxy<ℓcoh(several)⋅tanθx,y

In sum, given {Lz,LS and ℓcoh}, inequality  guides us in selecting the remaining SVE parameter Nw.  Inequality   can be viewed as a constraint on the propagation mesh, or on the inclination of the target that can be accurately modeled.  For a complex target, tanθx,y will be a complicated variable quantity, and it may not be necessary to satisfy inequality   everywhere, as long as the violating areas return insignificant power.  At this point, there is some lack of precision about the constraints   and  , due to the "(several)" factor and the potential complexity of tanθx,y.

# Comparison of theory with a numerical simulation using WaveTrain's  “PartiallyCoherentReflector” model

The case of the tilted rectangular plate and Gaussian spectral density, for which we worked out numerical details from theory in Section 2.7, can be set up in WaveTrain as a test of the PartiallyCoherentReflector (PCR) module.

Let us choose the following WaveTrain propagation parameters for our PCR test system:

- 0 turbulence,  λ=1 μm,  Z=50 km,  propagation grid dimension N=256 points, 
grid spacing dx=λz/N=1.40 cm (Fresnel criterion),  grid width N⋅dx=3.58 m

- Reflector square of size L×L=1 m×1 m, inclined from y-axis at θ=0, 10, 30, and 80 deg (see Figure 2).
This is implemented with two types of PCR input maps (WaveTrain “Grids”):
(a) A depth map for each θ:   di,j=yj⋅tanθ, where i,j span L , Lcosθ.
(b) A reflectance map for each θ:  Ri,j=1.0,  where  i,j span L , Lcosθ.

- The PCR module has an input parameter called “coherenceLength”.  This coherence length is a 1/e2 length, where the visibility (or correlation) function is Gaussian.  Therefore, the PCR “coherenceLength” should correspond precisely to  ℓcoh as defined in the preceding theoretically-based numerical example.  If we let ℓcoh= 0.50, 0.10, 0.02 m, then Equation  tells us that the specified combination of  ℓcoh and θ parameters should correspond to the range of NIV tabulated in Table 2.  Notice that the range of speckle contrast that we obtain from the parameter domain in the table is only one order of magnitude, while the range of NIV is two orders of magnitude.

|  |  |  | NIV (Contrast) |  |  |
| --- | --- | --- | --- | --- | --- |
|  (deg) | Lz = Lsin (m) | Ly = Lcos (m) | ℓcoh= 0.50 m | ℓcoh= 0.10 m | ℓcoh= 0.02 m |
| 0 | 0 | 1 | 1 (1) | 1 (1) | 1 (1) |
| 10 | 0.174 | 0.985 | 0.77 (0.88) | 0.23 (0.48) | 0.051 (0.23) |
| 30 | 0.500 | 0.866 | 0.38 (0.62) | 0.089 (0.30) | 0.018 (0.13) |
| 80 | 0.985 | 0.174 | 0.21 (0.46) | 0.045 (0.21) | 0.0090 (0.095) |

Table 2:  Expected range of speckle NIV spanned by the WaveTrain test inputs

- The final key input parameter in the PCR module is “nWaves”.  This is the parameter denoted Nw in the conceptual overview Section 3.  We performed test runs for Nw= 20 and 100.  In the discussion of results below, we will relate these values to the inequality requirements discussed at the end of Section 3.

- Sensor parameters in the WaveTrain receiver plane:  
We want to compute the speckle irradiance in the simulated observation plane for each combination of the test parameters.  To make a valid comparison between theory and simulation, we must bear in mind several artifacts that arise in the simulated Fourier transform propagation on a discrete, finite mesh.  
(i)  The first item to consider is peculiar to the case of propagation from an optically rough source plane, where the transverse phase has a spatially uncorrelated term distributed over (0, 2).  In a physical experiment this light would have a wide divergence, but on the discrete Fourier transform mesh the light from each mesh point only diverges by an amount limited by the propagation mesh spacing, as sketched in Figure 6. The consequence is that the scattering model is only valid in the overlap region indicated in Figure 6 by the segment labeled L uni:  in this region, all discrete source points that comprise the specified reflector contribute to the net light field, as they would in a physical experiment.  When we compute the speckle statistics, we must limit ourselves to a region of interest (ROI) in the receiver plane that is no wider than L uni.  From Figure 6, we deduce that the width of the uniform region is

Luni=(Z−Z0)⋅λdxgrid,  where Z0=Lx refl/λdxgrid ⇒ Luni=Zλdxgrid−Lx refl

Figure 6:  ROI over which average irradiance is uniform, indicated by Luni in the sketch

The smallest Luni occurs for the largest Lx refldimension, which is 1m in our test cases:  in that case, Z0=14km and Luni=2.57m.  We choose the ROI in the receiver plane be a square with LROI=1.5m.

(ii)  The second item to consider is this:  if we want to compute NIV spatially over the receiver plane, then LROI should contain many speckles.  Stated another way, LROI should contain many intensity correlation lengths.  If this condition is violated, we could still compute NIV temporally, across many independent simulation runs, but this would much more tedious.  The typical monochromatic speckle size that is expected is approximately

Lx,yspk=λLx,yZ

For our test parameter set, the two extreme situations are:
     (a)  At the 0-deg inclination angle, Lx, Lyrefl=1.0, 1.0 m, and Lx,yspk=0.05, 0.05 m
     (b)  At the 80-deg inclination angle, Lx, Lyrefl=1.0, 0.174 m, and Lx,yspk=0.05, 0.29 m.  
Therefore, in our 1.5 m×1.5 m ROI, we can expect roughly 30×30 speckles for case (a), and roughly 30×5 speckles for case (b).  There are actually two points to be made here.  As we noted initially, computing a spatial irradiance variance over the ROI should be unbiased in the sense that the ROI spans many correlation lengths.  Second, the estimation error for the irradiance variance depends on the number of independent samples, as well as on the irradiance PDF.  The number of independent samples may be somewhat marginal for the 30×5 speckles case, so it is probably desirable to repeat the WaveTrain propagations for, say, 8 realizations of the rough surface and pool the irradiance data to compute NIV.

## Comparison of theory and WaveTrain PCR simulation: results

Results for the two Nw cases are collected in  Figure 7 and Figure 8.  The solid red line in each figure is the theoretical curve previously plotted in Figure 3, and the symbols are PCR-simulated results for the various combinations of coherence length and plate tilt (where plate tilt controls the macro-depth of the target, as tabulated in Table 2).  The horizontal-axis plot variable is v=DκLz=4Lz/ℓcoh:  according to the theory in Section 2, the NIV should only depend on this dimensionless ratio.  Inspecting the plot for Nw = 100 (Figure 7), we see that the match between theory and PCR simulation is good except at the largest plate tilt of 80 deg.  These results are completely consistent with the PCR parameter guidelines developed at the end of Section 3 (inequalities   and  ).  For example, when ℓcoh = 2 cm and θ = 80 deg, inequality   tells us that we should have dxy < (0.35/several) cm.  The actual dxy of the propagation mesh used for all runs was 1.4 cm, a gross violation of the constraint, so as expected the agreement between theory and PCR simulation is poor for this parameter combination.  On the other hand,  when ℓcoh = 2 cm and θ = 30 deg, inequality   tells us that we should have dxy < (3.46/several) cm.  In this case, the actual dxy=1.40 cm marginally satisfies the constraint, and indeed we see in the plot that theoretical and simulated NIV agree fairly well.  Similar consistency checks can be made for the other data points in  Figure 7 and Figure 8.  The fact that the simulated results diverge from truth just when we begin to violate certain parameter constraints gives us further confidence that we correctly understand the operation of the simulation module.

Figure 7:  Theoretical and PCR-simulated results for NIV, for nWaves (Nw) = 100

Figure 8:  Theoretical and PCR-simulated results for NIV, for nWaves (Nw) = 20

# Summary

We have carried out a quantitative test of the WaveTrain module "PartiallyCoherentReflector" (PCR).  This rather complex module simulates the reflection of partially coherent illumination from an optically-rough reflector.  For an analytically tractable geometry, we developed a theoretical formula for the normalized irradiance variance (NIV) expected at a receiver plane after scattering from the rough surface.  We then simulated scattering from that geometry using the WaveTrain PCR.  Over the tested range of NIV, we found good agreement between theory and WaveTrain simulation, when validity constraints on the simulation parameters were satisfied.

Parry, G.:  "Speckle Patterns in Partially Coherent Light", in Laser Speckle and Related Phenomena, 2nd ed., J.C. Dainty (ed.), pp. 77 ff., Springer-Verlag, 1984.

J.W. Goodman,  Statistical Optics, p. 400, Wiley, 1985.

Frieden, , B.R.:  Probability, Statistical Optics, and Data Testing, 2nd ed., pp. 105 ff., Springer-Verlag, 1991.

Goodman, J.W.:  “Statistical properties of laser speckle patterns”, in Laser Speckle and Related Phenomena, J.C. Dainty (ed.), pp. 9 ff., Springer-Verlag, 1975.